# Liquid-Solid Chromatography (LSC)

Liquid-Solid Chromatography (LSC) is a type of liquid chromatography where the stationary phase is a solid and the mobile phase is a liquid.

## Principle and Mechanism:

  - The separation mechanism in LSC is based on the competition of the components of the mixture sample for the active sites on an adsorbent.

  - This is an adsorption chromatography process, involving distribution between two phases through adsorption and desorption.

  - Attractive forces such as ionic, dipole-dipole, and dipole-induced dipole interactions play a role.

  - Good adsorbents are characterized by a large surface area and more active sites.

  - Equilibration occurs at the surface-solute, surface-solvent, and solvent-solute interfaces.

- Adsorption Isotherms: An adsorption isotherm plots the concentration or amount of analyte on the adsorbent surface as a function of its concentration in the bulk phase (mobile phase). Types include:

  - Linear: where the partition coefficient (k = Cs / Cm) equals 1.

- Adsorbents: The adsorbing power of a material depends on its chemical nature, available surface area, and pretreatment. Commonly used adsorbents include:

  - Alumina (Al2O3): Can be acidic, basic, or neutral, and is available in various grades.

  - Silica gel (silicon dioxide): Acidic in nature and available in various grades; it has a high capacity and is useful for both adsorption and partition chromatography.

  - CaCO3.

  - Sucrose.

  - Starch.

  - Cellulose. Adsorbents should have uniform size and a large surface area. The weight of the adsorbent should typically be 20-50 times more than the sample.

- Solvents & Adsorbates:

  - Since adsorbents are generally polar, non-polar compounds elute first.

  - The typical elution order for adsorbates, from non-polar to polar, is: alkyl halides < saturated hydrocarbons < unsaturated hydrocarbons < ethers < esters < ketones < amines < alcohols < phenols < acids and bases.

  - Polymeric compounds and salts often do not elute.

  - Solvents are ordered by increasing polarity (elution strength): Hexane/Petroleum ether < CCl4 < toluene < dichloromethane < chloroform < diethyl ether < acetone < ethyl acetate < propanol < ethanol < methanol < acetic acid < water.

- Columns and Packing: Various sizes of columns are available, and packing can be done using either wet method or dry method.

- Example Application: LSC is used for separating water-soluble vitamins.